# PROJECT STATUS REPORT: Autonomous 5-Bar Parallel SCARA Manipulator

Date: September 7, 2026

## 1. Ultimate Project Objective

The overarching objective of this project is to engineer, program, and validate a fully autonomous, closed-loop 5-bar parallel SCARA robotic manipulator capable of visual targeting and physical object sorting.

The system bridges advanced mechanical kinematics with real-time computer vision to identify targets within a physical workspace, compute precise real-world cartesian coordinates, and execute pick-and-place operations using a specialized electromagnet end-effector.

To ensure research-grade physical precision, the ultimate iteration of this system integrates a strict closed-loop feedback architecture. By mapping dual AS5600 magnetic encoders to the stepper motor shafts, the microcontroller will continuously verify the mechanical execution of the Inverse Kinematics (IK), establishing a real-time error-correction loop that guarantees absolute end-effector positional accuracy.

## 2. Progress Update: Completed Milestones

The project has successfully transitioned through the foundational mechanical, electrical, and software phases. The system is currently fully operational in an open-loop state.

Previously:

- Kinematics & Motion Control (Firmware):

  - Successfully derived and embedded mathematical Inverse Kinematics natively onto the ESP32 to calculate complex joint angles in real time.

  - Configured NEMA 17 stepper motors with DRV8825 drivers, utilizing the FastAccelStepper library for high-speed, interrupt-driven trapezoidal motion profiling.

- Electrical & End-Effector Integration:

  - Successfully integrated a 5V DC Electromagnet (P25/20) with a ~5kg holding capacity.

  - Resolved a critical 3.3V-to-5V hardware logic-level mismatch on the control relay by implementing software-defined High-Impedance state switching (pinMode toggling).

  - Stabilized the magnetic coil with a dedicated 2A external power supply, optimizing magnetic flux dwell times for reliable payload securing.

Last 2 weeks:

- Computer Vision Pipeline (Software):

  - Deployed an OpenCV-based tracking pipeline over an IP Web camera stream, accurately transforming detected pixel data (u,v) into real-world cartesian coordinates (x,y).

  - Calibrated to a 0.875 mm/px scale.

- Full-Cycle Autonomy:

  - Established a robust serial "Handshake Protocol" between Python and the ESP32.

  - Implemented a visual auto-homing sequence (VHOME) that dynamically calibrates the manipulator's starting coordinates by visually locating a green fiducial marker on the end-effector.

  - Automated the comprehensive task loop (VPICK): Target identification, precision grasping, transport to a designated drop zone (X: 150, Y: 0), payload release, and safe retreat (X: 0, Y: 150).

UPLOAD THE VIDEO!!

## 3. Ongoing Tasks: The Closed-Loop Integration Roadmap

While the open-loop system functions smoothly, the manipulator currently relies blindly on the stepper motors executing their commanded step counts. The final phase of the project is entirely focused on hardware feedback and error correction.

### Immediate Objectives (Hardware & Wiring)

- Dual I2C Bus Configuration: Wire the two AS5600 magnetic encoders to the ESP32. Because the AS5600 has a hardcoded I2C address (0x36), the sensors will be wired to the ESP32's two independent hardware I2C buses (Bus 0: GPIO 25/27, Bus 1: GPIO 32/33) to prevent signal clashing without requiring a multiplexer.

- Baseline Sensor Validation: Write and execute standalone diagnostic firmware to ensure both encoders smoothly output 12-bit raw angle data (0–4095) across a full 360-degree rotation of the NEMA 17 shafts.

### Secondary Objectives (Software & Algorithms)

- Resolution Mapping: Develop the mathematical conversion matrix to bridge the gap between the NEMA 17's microstepping resolution (6,400 steps per revolution at 1/32 mode) and the AS5600's fixed magnetic tick resolution (4,096 ticks per revolution).

- Closed-Loop Feedback Implementation:

  - Modify the main execution loop to read the starting AS5600 angles prior to movement.

  - Command the physical step sequence derived from the IK calculations.

  - Read the post-movement AS5600 angles, convert the raw data into degrees, and compare it against the IK target angles.

  - Calculate the discrepancy (error margin) and command corrective micro-steps if the end-effector falls short of the target coordinate.

### Remaining Tasks:

- 3d print the workspace setup and fully mount the system

- Calibration

- System Benchmarking: Run extensive pick-and-place cycles under varying speeds and payloads to document the correction latency and overall positional accuracy of the closed-loop system.

- Documentation: Finalize code commenting, GitHub repository archiving, and compile the final thesis/project report.

Git repo:

https://github.com/tanvir-alam-mollah/Parallel-SCARA-vision-robot.git